# PCB production guideline – Single PCB

| PCB Label | EVK75123-Sensor-MLX75024 V1.0 |
| --- | --- |
| Dimension (l x b) | 80 x 50 mm ±0,2mm |
| Material | FR4 |
| PCB Thickness | 1.2 mm ±10% |
| Outline | milled scored |
| Surface Coating | HAL (Pb free) chem. Ni/Au |
| Copper Thickness | 35µm |
| Layer Stack | 8 layer |
| Minimum Track Width | 0.1mm |
| Minimum Clearance | 0.1mm |
| Smallest Drill Size (mech.) | 0.1 mm |
| Solder Resist | green (both sides) |
| Component Print | only on top layer (.gto) |
| Additional Prints | no |
| Test | optical elektrical |
| Number of SMD Pads | 791 |
| SMD Assembling | top and bottom |

Additional Information	All vias with diameters ≤ 0.3 mm must be plugged and plated with copper.
All drillings > 0.3 mm must stay open

Differential signals that are routed with a trace width of 0.12 mm and a separation of 0.1 mm should match an impedance of 100 Ω ± 10%. The purposed layer stack on page 3 can be adapted by the manufacturer in consultation with BECOM Bluetechnix.

# Gerber File Information

| Kupfer Lage | Gerber File Extension | Lagen Beschreibung |
| --- | --- | --- |
| 1 | .GTL | Top Layer |
| 2 | .G1 | Internal Layer 1 |
| 3 | .GP1 | Plane Layer 1 |
| 4 | .GP2 | Plane Layer 2 |
| 5 | .GP3 | Plane Layer 3 |
| 6 | .GP4 | Plane Layer 4 |
| 7 | .G2 | Internal Layer 2 |
| 8 | .GBL | Bottom Layer |
|  | .GTO | Top Overlay |
|  | .GTP | Top Paste |
|  | .GTS | Top Solder |
|  | .GBO | Bottom Overlay |
|  | .GBP | Bottom Paste |
|  | .GBS | Bottom Solder |
|  | .GM1 | Boardoutline |

| NC-Drill File Extension | Lagen Beschreibung |
| --- | --- |
| .txt | Drill Top to Bottom |

# Layerstack

The following layer stack should match the criteria for the desired differential impedance ant the 0.1 mm via drilling. The manufacturer can make adaptions in consultation with BECOM Bluetechnix.